This is the documentation file for TESTRECS.ZIP.  TESTRECS.ZIP contains

three test recordings from the Gravis UltraSound soundcard.  I made these

recordings to compare the quality of three different recording programs:

USS8, PLAYFILE, and GUSDELAY (a new, unreleased version).  This new,

soon-to-be released version of GUSDELAY uses a revolutionary new recording

technique that I developed (so I may be a little biased when I say

"revolutionary") which allows jitter-free and dropout-free recording with

the GUS.

Each recording was made using the same test conditions.  The only difference

in the three tests was the software used for recording.  Here is a breakdown

of my test setup.

Computer: 25 MHz 386DX clone w/ 32 KB cache and 4 MB ram.

Bus Speed: 8 MHz

Hard Drive: Seagate ST-1096N (I think). 80 MB SCSI

SCSI Adapter: Seagate ST-01 (or ST-02, I can't remember). 8-bit only!

GUS DMA Setup: Record DMA on channel 5, Playback DMA on channel 6

Test Signal Source: (don't laugh) Casiotone MT-800's pipe organ sound.  The

note used is the C above middle C, but the keyboard is probably way

out of tune.

Test Signal Level: Line level.

Sampling Frequency: 44100 Hz

Channels: 1 channel (i.e. mono recording)

Notes: I fully defragmented the hard drive before each recording.  I used

SOX version 7 to create a .WAV file from the headerless data files of USS8

and GUSDELAY.  The PLAYFILE recording was created as a .WAV file (using the

-wav switch on the command line).  The recordings are not all the same

length, but they are all approximately 5 seconds in length.

US44100.WAV was recorded using USS8.  This is the worst of the recordings.

Clicks can be distinctly heard at regular intervals.  I assume these are due

to missed samples and/or "jitter".

PF44100.WAV was recorded using PLAYFILE.  This sounds significantly better

than US44100.WAV, but several clicks can still be heard (although they are

more subtle that the clicks in US44100.WAV).  I assume these are also due to

missed samples and/or "jitter".

GD44100.WAV was recorded using GUSDELAY.  This is the best recording of the

three.  There are no clicks whatsoever.  This is due to the revolutionary

new recording technique that I developed.

One thing that I haven't tried yet is to use SOX to maximize the volume of

these sounds.  I think that will make the differences in quality even more

noticeable.

I am working on a document that describes this new recording technique.  It

should be done in a few days.

Please let me know what you think about the recordings.  I am interested in

the opinions of unbiased ears.

I can be reached via internet e-mail.  My address is davidm@marcam.com.

Dave

David MacMahon

Systems Administrator

davidm@marcam.com