BACKPACK CD-ROM with PC Card User’s Guide

rev 07.99.01

Getting Started

To get started with BACKPACK first install the software driver from the diskette. After you restart your computer it will be ready to use the BACKPACK attached to the BACKPACK PC Card or to any parallel printer port (LPT port).

Software Installation

Place the BACKPACK SETUP diskette in your A: drive.

Click Start, Run. In the dialog box enter:

A:SETUP <Press OK>

Follow the prompts that appear.

The BACKPACK software driver should now be installed on your system. You must restart the computer before using the BACKPACK drive.

Hardware Installation

Insert the BACKPACK PC Card into your computer.  If your computer does not have PC Card capabilities, connect the BACKPACK drive to your computer’s parallel port (LPT Port).

Plug the BACKPACK power unit into a wall outlet and attach the power cable to the power connector on the BACKPACK drive.  (BACKPACK does not receive power from the BACKPACK PC Card).

Connect the cable from the PC Card to the port marked "Computer” on the BACKPACK drive.  You can also connect BACKPACK to the printer port connector on your computer. A separate cable is provided for this purpose. When connected to a printer port you can connect a printer to the BACKPACK port marked "Printer”.

After a moment the BACKPACK drive will be assigned a drive letter and be ready to use.

IMPORTANT COMPATIBILITY NOTES

- In DOS or Windows 3.x:

BACKPACK can only be connected to the computer's parallel port (LPT Port).  The BACKPACK will not work on the PC Card under DOS or Windows 3.x.  Plug and play is not supported in DOS or Windows 3.x.  BACKPACK must be connected to the computer's parallel port before starting the computer.

- In Windows 95 or Windows 98:

BACKPACK can be connected to either the computer's parallel port (LPT Port), or to the BACKPACK PC Card.  Plug and play compatibility is provided on both the parallel port and the BACKPACK PC Card.

- In Windows NT 4:

BACKPACK can only be connected to the computer's parallel port (LPT Port).  The BACKPACK will not work on the PC Card under Windows NT 4.  Plug and play is not supported in Windows NT 4.  BACKPACK must be connected to the computer's parallel port before starting the computer.

- Printer compatibility:

Printers can only work on the BACKPACK's pass-through "Printer" port when the BACKPACK is connected to the computer's parallel port (LPT Port).  Printers cannot be used on the BACKPACK PC Card.

The BACKPACK PC Card is only designed to support BACKPACK drives.

Removing BACKPACK Software

To remove the BACKPACK drivers use Add/Remove Programs in Control Panel.

Troubleshooting

BACKPACK's Drive Letter

When BACKPACK is connected Windows will assign it the next available drive letter. This could be the letter D, E or any other letter - depending on the letters already in use on your computer.

Open Explorer or My Computer and look for an icon similar to a CD-ROM disc.

BACKPACK Did not receive a drive letter:

Check for Real-Mode CD-ROM Drivers in Windows 95 or Windows 98

You computer could have older 16-bit drivers installed for another CD-ROM. These will interfere with BACKPACK.

Check the system AUTOEXEC.BAT file and see it the program MSCDEX.EXE appears. Disable that line by placing the word "REM" at the beginning. If your pre-existing CD-ROM no longer receives a drive letter, contact your CD-ROM manufacturer to obtain Windows 95 or 98 -compatible 32-bit software for your pre-existing CD-ROM.

TIP:  You can view or edit AUTOEXEC.BAT by clicking Start, Run and typing "SYSEDIT".

Check BACKPACK Properties

BACKPACK settings can be examined or changed from within programs installed in the Windows Control Panel.

If you click Start, Settings, Control Panel you will see an icon for BACKPACK Finder (Windows 95 and 98) or BACKPACK (Windows NT).  If you're not sure, Open Control Panel and see which icon is installed.

BACKPACK Finder and BACKPACK Controls are discussed below.

BACKPACK Finder (Windows 95 & 98):

Open the BACKPACK Finder icon located in Windows' Control Panel:

The upper pane under Devices shows:

		Printer ports or a PCCARD adapter that a BACKPACK drive might be connected to,

for example LPT1, LPT2, PCCARD1, or PCCARD2.

		Beneath each port or adapter will appear any BACKPACK drives that have been detected.

No LPT ports or PCCARDs detected in BACKPACK Finder

		Click Update in the "Devices" screen and see if the LPT port or PC Card appears.

		The BACKPACK software drivers may not be properly installed. Try repeating the software setup.

		If the BACKPACK PC Card is not detected you may need to enable 32-bit PC Card support in Windows.

Open Windows Help and refer to PC Card in the Index.

		Review the Troubleshooting PC Card section of Windows Help

No BACKPACK Devices Detected in BACKPACK Finder

		Check the connections and verify that the BACKPACK is powered on.

- Try disconnecting and reconnecting the BACKPACK drive from the PC Card or printer port.

		Click Update in the "Devices" screen and see if a drive appears.

- There are "Finder Settings" that you can change that may correct the problem also.  These options are detailed next.

BACKPACK settings within the BACKPACK Finder utility

The following changes can be made in BACKPACK Finder utility program located in the Windows Control Panel. Try these changes one at a time, in the order listed below. Restart your computer after making the change and see if it corrects the problem.

Clear the Enable Interrupts checkbox.

Clear the Enable I/O Bus Speed Control checkbox.

Clear the "EPP ..." checkboxes.

After changing this value (EPP), power off both the computer and the BACKPACK to fully reset the system.

Clear the Detect LPT Port Type checkbox.

Uncheck the ECP + DMA checkbox.

Default Settings in BACKPACK Finder:   All boxes selected.

If a change does not resolve the issue, set it back to the default and try the next option. Only in rare instances are a combination of changes needed.  Combinations of these are valid as well, but not typically necessary.

BACKPACK Controls (Windows NT4):

Open the BACKPACK icon located in the Windows NT4 Control Panel.  The large box on the right displays information about the computer's ports and the BACKPACK drive detected by the software.

No BACKPACK(s) Detected during boot process!

	Check the connections and verify that the BACKPACK is powered on.

	BACKPACK must be connected and powered on when the computer starts.

- There are settings in BACKPACK Controls that you can change that may correct the problem also. These options are detailed next.

BACKPACK Settings in the BACKPACK Controls utility

The following changes can be made in BACKPACK Controls. Try these changes one at a time, in the order listed below. Restart your computer after making the change and see if it corrects the problem.

- IRQ from Auto to Off

- BUS from Auto to Off

- MAX from EPP to Bi (or ECP)

After changing this value (EPP), power off both the computer and the BACKPACK to fully reset the system.

- Clear the  Test for Chipset checkbox.

- DMA from Auto to Off

- T1 from Auto to 10, 20 or higher

If a change does not resolve the issue, set it back to the default and try the next option. Only in rare instances are a combination of changes needed.  Combinations of these are valid as well, but not typically necessary.

Default Values

Default settings in BACKPACK Controls:

T1, T2, T8 = AUTO

MAX = EPP

BUS = AUTO

IRQ = AUTO

DMA = AUTO

Test For Chipset Checked

Auto Insert Notification Checked

BACKPACK in DOS or Windows 3.x

Detailed troubleshooting steps for BACKPACK CD-ROM in DOS or Windows 3.x is located online at Micro Solutions' Website (see Technical Support below) and in the downloadable files CDGUIDE.DOC and CDGUIDE.TXT (available online in the Software Library section).  BACKPACK drive letter assignments, detection difficulties, and MSCDEX options are covered in those documents.

Printer Troubleshooting

		If you are using the BACKPACK PC Card, you cannot use a printer connected to the pass-through "Printer" 	port on the back of the BACKPACK. The BACKPACK "Printer" connection will only work if BACKPACK itself is plugged into the computer's printer port (LPT port).

		Many printing problems can be solved by turning off Auto Insert Notification (detailed below) or by turning off print spooling in Windows (detailed below).

Turning off Auto Insert Notification in Windows 95 & 98

		Open Device Manager (Select Start, Settings, Control Panel, System, Device Manager)

		Choose "BACKPACK" under the CDROM category.

- Choose "Settings" and uncheck the Auto Insert Notification option.

- Restart the computer.

Turning off Auto Insert Notification in Windows NT 4

		Open Control Panel (Select Start, Settings, Control Panel)

		Double-click the BACKPACK icon

- Uncheck the Auto Insert Notification option.

- Restart the computer.

Turning off print spooling in Windows 95 & 98

		Select Start, Settings, Printers. Right-click your printer.

		Select Properties, Details. Click Spool Settings.

		If available, select Disable bi-directional support for this printer and select "Print directly to the printer."

- Select Port Settings.

- Disable "Check port state before printing."

- Restart the computer.

Turning off print spooling in Windows NT 4

		Select Start, Settings, Printers. Right-click your printer.

		Select Printer, Properties, Scheduling.

		Select Print Directly to the Printer.

- Restart the computer.

Technical Support

Micro Solutions via internet

BACKPACK software is being continually improved. Often problems can be solved by installing the latest drivers.

Software drivers and technical Support documents are available on the Web at:

http://www.micro-solutions.com.

Micro Solutions technical help by FAX

You can have technical support information sent directly to your FAX machine.

Contact Micro Solutions' Automated Fax Response at 815.754.4600. Automated Fax Response is available 24-hours daily.  A complete catalog of faxes and information is available.

NOTE: If you are calling from outside the United States first enter the digits 011 when you are asked to enter your country code and FAX number.

Most questions about BACKPACK and its operation are answered in this guide (PCGUIDE). To solve most problems:

	Check the solutions and procedures in this guide.

	Check the README.TXT files on the BACKPACK installation diskette.

	Visit Micro Solutions on-line at our Web site at:

http://www.micro-solutions.com.

	Contact Micro Solutions' Automated Fax Response at 815.754.4600.

If your problem remains unsolved, contact Micro Solutions Technical Support Department at 815.754.4500. Technical Support is available during normal business hours, Monday through Friday, Central Time. Before calling, be sure to have the following information ready:

	The version numbers of your BACKPACK software and your operating system (Windows 95, 98 or NT).

	The name and model of the computer, and the eight-digit serial number found on the bottom of your BACKPACK drive.

	The exact wording of any error message(s) from the BACKPACK program, DOS, Windows or any other application producing the error message.

If possible, be at your computer when calling.