TIARA Computer Systems, Inc.

(C) Copyright 1992.  All Rights Reserved.

---------------------------------------------------------------------------

NDIS Device Driver Installation

- for -

Microsoft Lan Manager Versions 1.10 and 2.00

---------------------------------------------------------------------------

Please note that these instructions assume that the user is already

familiar with the requirements of the particular environment in terms of

workstation setup and software generation.  That is, these instructions

only deal with the basic issue of merging the TIARA device driver into the

particular environment so that workstation generation may then be performed

by the user.

Copyright/Trademark Information:--

Microsoft is a registered trademark of Microsoft Corporation.

OS/2 is a registered trademark of International Business Machines

Corporation.

General Instructions:--

In the instructions which follow, "S:" is used to designate the

(SOURCE) drive containing the TIARA driver distribution diskette,

and "T:" is used to designate the (TARGET) drive to which the

particular item is to be installed.

For example, if the TIARA diskette is in drive "A:" and the user

is installing to drive "B:", then the user should substitute "A:"

for "S:" and "B:" for "T:" as follows:

Original command:   "COPY S:FILENAME T:"

Modified command:   "COPY A:FILENAME B:"

Installation instructions for version 2.00

The MSLANMAN.DOS and MSLANMAN.OS2 directories have been constructed in

accordance with the requirements of the Lan Manager SETUP utility

Action menu Import function.

___________________________________________________________________________

Document:	LANMAN.DOC	    June 12, 1992			  Page 1

TIARA Computer Systems, Inc.

(C) Copyright 1992.  All Rights Reserved.

---------------------------------------------------------------------------

NDIS Device Driver Installation

- for -

Microsoft Lan Manager Versions 1.10 and 2.00

---------------------------------------------------------------------------

Installation instructions for version 1.10

OS/2 diskette preparation.

1.   Use the DISKCOPY command to make a duplicate of the OS/2

NET TRANSPORTS & NDIS DRIVERS diskette for steps 2 and 3.

2.   Create the TIARA device driver directory:

MD T:\LANMAN\DRIVERS\TNDIS

3.   Copy the TIARA driver files to the above directory:

COPY S:\MSLANMAN.OS2\DRIVERS\ETHERNET\TNDIS\TNDIS.OS2

T:\LANMAN\DRIVERS\TNDIS

COPY S:\MSLANMAN.110\OS2\TNDIS.NIF T:\LANMAN\DRIVERS\TNDIS

COPY S:\MSLANMAN.110\OS2\PROTOCOL.INI

T:\LANMAN\DRIVERS\TNDIS

DOS diskette preparation.

1.   Use the DISKCOPY command to make a duplicate of the DOS NET

TRANSPORTS & NDIS DRIVERS diskette #2 for steps 2 and 3.

2.   Create the TIARA device driver directory:

MD T:\LANMAN\DRIVERS\TNDIS

3.   Copy the TIARA driver files to the above directory:

COPY S:\MSLANMAN.DOS\DRIVERS\ETHERNET\TNDIS\TNDIS.DOS

T:\LANMAN\DRIVERS\TNDIS

COPY S:\MSLANMAN.110\DOS\TNDIS.NIF T:\LANMAN\DRIVERS\TNDIS

COPY S:\MSLANMAN.110\DOS\PROTOCOL.INI

T:\LANMAN\DRIVERS\TNDIS

___________________________________________________________________________

Document:	LANMAN.DOC	    June 12, 1992			  Page 2

TIARA Computer Systems, Inc.

(C) Copyright 1992.  All Rights Reserved.

---------------------------------------------------------------------------

TNDIS PROTOCOL Parameter Descriptions

---------------------------------------------------------------------------

The following parameters are generic to any model of the EtherStar-based

LanCard/E.

KeyWord       Minimum Maximum Default   Description / Comments

------------  ------- ------- -------   -----------------------------------

DriverName                     TNDIS$   REQUIRED Parameter.

Up to nine copies of this driver

may be loaded into memory at once,

one for each LanCard/E supported.

The default name specified will

apply to the first copy of the

driver.  Additional copies should

use the following names in the

order specified:

TNDIS2$, TNDIS3$, TNDIS4$, TNDIS5$,

TNDIS6$, TNDIS7$, TNDIS8$, TNDIS9$

MaxTransmits        1     142       6   Maximum number of Transmit Requests

which can be queued by the driver.

The following parameters are specific to the particular version of

LanCard/E.

ISA-bus:  Industry Standard Architecture.

MCA-bus:  Micro Channel Architecture.

KeyWord       Minimum Maximum Default   Description / Comments

------------  ------- ------- -------   -----------------------------------

Interrupt           2      15       3   Interrupt Request Level for ISA-bus

machines.

For allowable values, consult the

appropriate owner's manual.

IOAddress           0 0x0FFE0   0x300   Input/Output Base Address for ISA-

bus machines

For allowable values, consult the

appropriate owner's manual.

___________________________________________________________________________

Document:	LANMAN.DOC	    June 12, 1992			  Page 3

TIARA Computer Systems, Inc.

(C) Copyright 1992.  All Rights Reserved.

---------------------------------------------------------------------------

TNDIS PROTOCOL Parameter Descriptions

---------------------------------------------------------------------------

KeyWord       Minimum Maximum Default   Description / Comments

------------  ------- ------- -------   -----------------------------------

Slot                0       8       0   Slot Number for MCA-bus machines.

If the Slot Number is set to zero,

or unspecified on a MCA-bus

machine, the driver will execute a

search algorithm scanning from slot

1 to 8 in an attempt to identify a

slot containing a LanCard/E*MC.

If the above search is

unsuccessful, or the specified slot

does not contain a LanCard/E*MC,

the driver will use the IOAddress

and Interrupt parameters just as

though it were operating in an ISA-

bus machine.

___________________________________________________________________________

Document:	LANMAN.DOC	    June 12, 1992			  Page 4