TIARA Computer Systems, Inc.

(C) Copyright 1992.  All Rights Reserved.

---------------------------------------------------------------------------

LanCard Configuration and Diagnostic Utility

---------------------------------------------------------------------------

TIARA Computer Systems provides extended software features with its

network interface products by supporting board configuration and

functionality testing via the DIAG diagnostic utility.

To run this utility, type "DIAG [-b] <enter>".  -b is an optional switch

that changes the video I/O mode from direct I/O mode to BIOS access mode.

The -b switch is required only if running without it produces a blank

screen.  Should this occur, exit the program by hitting the escape key

and rerun DIAG with the -b option.

Configuration.

The configuration portion of the utility displays an image of the

adapter while prompting the installer through configuration options.

Functionality Testing.

Prior to running the DIAG.EXE diagnostic program, make sure that no

adapter drivers have been loaded.  Attaching a cable to the adapter is

optional when running the diagnostic.

The loopback test portion of this utility provides a simple procedure

for validating that the card does perform all internal functions once

installed into the target machine.

This test should be run after the installation and configuration of the

adapter is complete.  During the test, no data is transmitted over the

network, thereby insuring data integrity.

If an error message is received during or after the test, then recheck

the adapter settings and try again.

___________________________________________________________________________

Document:  DIAGS.DOC        July 31, 1992                            Page 1

Revised May 3, 1993