TIARA Computer Systems, Inc.

(C) Copyright 1992.  All Rights Reserved.

---------------------------------------------------------------------------

Novell NetWare 286/386 Driver Installation

---------------------------------------------------------------------------

This file describes the steps needed for installation of the TIARA NetWare

device driver(s) in the following environments:--

NetWare 286 Versions 2.12 and 2.15c

NetWare 386 Versions 3.0 and 3.1x

Please note that these instructions assume that the user is familiar with

the requirements for Server and Workstation installation per Novell's

instructions for standard NetWare 286/386.

Files which pertain to NetWare 286 Installations:--

Filename	       Description

----------	       -------------------------------------------------

ATIARA.OBJ	       Object file for Server/Bridge Generation (LANA)

BTIARA.OBJ	       Object file for Server/Bridge Generation (LANB)

CTIARA.OBJ	       Object file for Server/Bridge Generation (LANC)

DTIARA.OBJ	       Object file for Server/Bridge Generation (LAND)

TIARA.LAN	       Configuration file for Server/Bridge Generation

STIARA.OBJ	       Object file for Shell (IPX) Generation

STIARA.LAN	       Configuration file for Shell (IPX) Generation

General Instructions:--

In the instructions which follow, "S:" is used to designate the

(SOURCE) drive containing the TIARA driver distribution diskette,

and "T:" is used to designate the (TARGET) drive to which the

particular item is to be installed.

For example, if the TIARA diskette is in drive "A:" and the user

is installing to drive "B:", then the user should substitute "A:"

for "S:" and "B:" for "T:" as follows:

Original command:   "COPY S:FILENAME T:"

Modified command:   "COPY A:FILENAME B:"

___________________________________________________________________________

Document:	NETWARE.DOC	   October 9, 1992			  Page 1

TIARA Computer Systems, Inc.

(C) Copyright 1992.  All Rights Reserved.

---------------------------------------------------------------------------

Novell NetWare 286/386 Driver Installation

---------------------------------------------------------------------------

Installation instructions for NetWare version 2.1x.

Note that the TIARA distribution diskette has been given a DOS volume

label of "LAN_DRV_223" which conforms to the requirements of the

ELSGEN/NETGEN/BRGEN/SHGEN utilities.

Floppy Disk Generations:

1.   Proceed according to the NetWare instructions for the

particular type of generation.

2.   At the LAN driver selection menu, for the first (or only)

driver, choose the "Load and Select" option and insert the

TIARA distribution diskette (LAN_DRV_223) when prompted to

do so.

3.   Respond to any informational or warning prompts as they

occur.

In normal circumstances, the only message/prompt which

should appear is the one which informs the user of the

Lan Option Name of the loaded TIARA driver(s).

First-time Hard Disk Generations:

1.   Follow the NetWare instructions pertaining to initial

upload of the NetWare diskette(s).

2.   When asked the question "Upload Additional Diskettes?"

answer yes, and insert the TIARA megadiskette when

requested to do so.

Pre-existing Hard Disk Generations:

The following instructions assume that the current directory is

set to the NetWare generation directory.

1.   Create the TIARA directory:

MD LAN_DRV_.223

2.   For a NetWare Server or Bridge generation, copy the

following files to the TIARA directory (LAN_DRV_.223):

ATIARA.OBJ, BTIARA.OBJ, CTIARA.OBJ, DTIARA.OBJ, and

TIARA.LAN

3.   For a NetWare Shell (IPX) generation, copy the following

files to the TIARA directory (LAN_DRV_.223):

STIARA.OBJ, and STIARA.LAN

___________________________________________________________________________

Document:	NETWARE.DOC	   October 9, 1992			  Page 2

TIARA Computer Systems, Inc.

(C) Copyright 1992.  All Rights Reserved.

---------------------------------------------------------------------------

Novell NetWare 286/386 Driver Installation

---------------------------------------------------------------------------

Installation instructions for NetWare 386.

Please Note:  TIARA strongly recommends that all modifications

outlined below be made to DUPLICATE copies of the stated diskettes

and NOT to the originals.

Version 3.0 Server

1.   Copy NETWARE.386\LANCARDE.LAN to the Netware 386 "SYSTEM"

diskette.

2.   Follow the instructions in the Novell Installation Manual

to load the driver.

Version 3.0 DOS Workstation

1.   Copy NETWARE.386\LANCARDE.COM to the master workstation

diskette.

2.   Follow the instructions in the Novell Installation Manual

to load the driver.

Version 3.1x Server

1.   Copy NETWARE.386\LANCARDE.LAN to the Netware 386 "Operating

System-2" diskette.

2.   Follow the instructions in the Novell Installation Manual

to load the driver.

Version 3.1x DOS Workstation

1.   Copy NETWARE.386\LANCARDE.COM to the master workstation

diskette.

2.   Follow the instructions in the Novell Installation Manual

to load the driver.

Requester for OS/2 Workstation

1.   If Novell's OS/2 Requester is not already installed,

install it now.

After the installation but BEFORE rebooting:

2.  Edit CONFIG.SYS to replace the current hardware driver with

this one.

Example:  old driver - device=c:\netware\ne2000.sys

replace with - device=c:\netware\lcarde.sys

3.  Copy LCARDE.SYS from the floppy to the directory specified

in the config.sys load statement.

Example:  copy a:lcarde.sys c:\netware\lcarde.sys

___________________________________________________________________________

Document:	NETWARE.DOC	   October 9, 1992			  Page 3

TIARA Computer Systems, Inc.

(C) Copyright 1992.  All Rights Reserved.

---------------------------------------------------------------------------

Novell NetWare 286/386 Driver Installation

---------------------------------------------------------------------------

4.  Edit NET.CFG, if necessary.	Include a section in NET.CFG

if you are currently using any special keywords with the

old driver.

Example:  old driver - link driver NE2000

frame ethernet_802.2

add - link driver LCARDE

frame ethernet_802.2

Note:  If you have set your hardware configuration jumpers

to something other than the default settings (IRQ 3, port

300), NET.CFG MUST be modified.

Example:  if your hardware is set to IRQ 5 and port 320h

add the following statements to NET.CFG:

link driver LCARDE

port 320

int 5

5.  Reboot your computer.

Industry Standard Architecture (ISA) Configuration options supported:

I/O Base Address	   300h, 320h, 380h, 3E0h,

260h, 280h, 2A0h, 2E0h

Interrupt Request        3, 4, 5, 7, 9, 10, 11, 12, 15

Note that for the LanCard/E*PC, interrupt

request jumper 2 is the same as interrupt

request 9 to the driver.

Node ID:                 Configurable.

Protocol ID:		   Ethernet_802.3 (default), Ethernet_802.2,

Ethernet_II, Ethernet_SNAP.

Micro Channel Architecture (MCA) Configuration options supported:

Slot Number:             1, 2, 3, 4, 5, 6, 7, 8.

Node ID:                 Configurable.

Protocol ID:		   Ethernet_802.3 (default), Ethernet_802.2,

Ethernet_II, Ethernet_SNAP.

___________________________________________________________________________

Document:	NETWARE.DOC	   October 9, 1992			  Page 4