Daniela's generic ATAPI Filter - Release 0.2.0

----------------------------------------------

(Birthday Edition)

NAME

DaniATAPI.FLT  -  replacement for IBMIDECD.FLT, IBMATAPI.FLT, DaniADsk.FLT

and all other ATAPI related filters.

SYNOPSIS

This filter driver supports all types of ATAPI devices. It is a superset

of IBMIDECD.FLT, IBMATAPI.FLT and DaniADsk.FLT, and supports all devices

that the mentioned drivers do.

Additionally, all ATAPI devices can be accessed through the OS/2 ASPI

software interface. Each EIDE channel shows up as a SCSI channel, and

each ATAPI device as a SCSI device (master = SCSI ID 0, slave = SCSI ID 1).

DESCRIPTION

This release of my ATAPI filter driver is based on the IBMATAPI.FLT and

IBMIDECD.FLT sources.

My enhancements are:

- can deal with the ATAPI ZIP-250

- supports arbitrary swapping of 100MB and 250MB media

- reflects the current media capacity up to FDISK, DISKIO or the like

- OS/2 boot from ZIP is possible

- faster 32bit PIO in conjunction with DANIS506.ADD (rel. 1.1.1 or later)

***  - ATAPI devices appear to the rest of the system as regular SCSI-2 devices

if used with DANIS506.ADD (rel. 1.1.5 or later).

***  - DMA mode works and is tested with Intel, VIA, AMD, HPT and SiS chipsets

until now. Due to hardware limitations there is no 32bit PIO and no

busmaster DMA possible for ATAPI devices attached to a Promise Ultra

controller.

***  - advanced "hot-swap" support if used with DANIS506.ADD (rel. 1.1.9 or

later)

This means, all freeware utilites designed to work with SCSI devices

are supposed to work with ATAPI devices as well:

CDRecord, CDDA2WAV, TAR, ...

COMMAND LINE OPTIONS

This driver supports all switches and options that IBM1ATAPI.FLT supports.

To enable booting OS/2 from an ATAPI ZIP drive, I introduced the new

/ZA switch ('map ZIP to A:'). It takes effect only if the BIOS has mapped

the ZIP to drive letter A: in order to boot from it; in all other cases

the /ZA option is a no-op.

If there are floppy disk drives in your system, then - as a side effect -

/ZA remaps floppy drive A: to B:, and floppy drive B: (if present) is

no longer accessable. Due to limitations in OS2DASD.DMD (?) floppy

reads and writes are a little 'rocky' in this case.

A boot from ZIP automatically locks the boot medium in the drive, media

swapping is no longer possible! This lock is imposed automatically by

OS2DASD. If you insist to eject the locked medium by the 'eject' command,

this command obviously is routed into nirvana and causes a kernel trap.

Units marked as "hot-swappable" on DANIS506.ADD's command line (by

/FORCE /ATAPI) need the new option /TYPE: to advice DaniATAPI.FLT to

report appropriate device types to the device managers. This is done

by additional characters designating the supported device types:

'c' or 'C': all CD-ROM like devices (CD-ROM, CD-R/W, DVD-ROM, DVD-RAM)

'z' or 'Z': all ZIP drives

'l' or 'L': the LS-120 drive

There may be more in the future. Some examples on how to use this option:

1) You have an IBM ThinkPad 600 and intend to plug a floppy drive, a

CD-ROM, and a ZIP-100 into its UltraBay. Then you need

BASEDEV=IBM1FLPY.ADD /A:0 /FORCE:1 /U:0 /F:1.44MB

BASEDEV=DaniATAPI.FLT /A:x /U:y /TYPE:ZC

2) You have notebook which can be equipped with either a LS-120 drive,

a DVD-ROM, or a CD writer. Then you need

BASEDEV=DaniATAPI.FLT /A:x /U:y /TYPE:CL

INSTALLATION

Just copy DaniATAPI.FLT to bootdrive:\OS2\BOOT and add the line

BASEDEV=DaniATAPI.FLT

Make sure that all other ATAPI related drivers are inactive or come

after DaniATAPI.FLT in your CONFIG.SYS.

If you prefer chasing icons, you can do the copy using DDINSTAL; you

still have to tidy up CONFIG.SYS!

If you intend to run the BootOS utility or to install this filter driver

on a 8.3-only filesystem, I recommend to rename the driver to DaniATAP.FLT.

FILES

DaniAATAPI.FLT  The driver itself.

DaniATAPI.DDP  The device driver profile for DDINSTAL

DaniATAPI.DOC  The file you are reading right now :-)

cmprssd\DaniATAPI.FLT

The same as above, but LXlite compressed. This one is *not*

guaranteed to work on every system.

BUGS

Hopefully none, but who knows ...

TO DO

- merge with the base S506 driver

COPYRIGHT

DaniATAPI - Copyright (c) 2000 Daniela Engert. All rights reserved.

STATUS OF PROGRAM

DaniADsk is freeware.

DISCLAIMER

YOU ARE USING THIS PROGRAM AT YOUR OWN RISK! I don't take any

responsibillity for damages, problems, custodies, marital dis-

putes, etc. resulting from use, inability to use, misuse, pos-

session or non-possession of this program directly or indirectly.

I also don't give any warranty for bug-free operation, fitness for

a particular purpose or the appropriate behaviour of the program

concerning animals, programers and little children.

THE SOFTWARE IS PROVIDED ``AS IS'' AND WITHOUT ANY EXPRESS OR

IMPLIED WARRANTIES, INCLUDING, WITHOUT LIMITATION, THE IMPLIED

WARRANTIES OF MERCHANTABILITY AND FITNESS FOR A PARTICULAR

PURPOSE. THE ENTIRE RISK AS TO THE QUALITY AND PERFORMANCE OF THE

PROGRAM IS WITH YOU.  SHOULD THE PROGRAM PROVE DEFECTIVE, YOU

ASSUME THE COST OF ALL NECESSARY SERVICING, REPAIR OR CORRECTION.

Or, in a few words:

If its good, I am responsible.

If its bad, its all your fault. ;)

Permission is granted to redistribute this program free of charge,

provided it is distributed in the full archive with unmodified

contents and no profit beyond the price of the media on which it

is distributed is made. Exception to the last rule: It may be

included on freeware/shareware collections on CD-ROM, as well as

on magazine cover CD-ROMs.

All trademarks mentioned anywhere around her are property of their

owners and the like ...

AUTHOR

Daniela Engert

Internet: "Daniela Engert" <dani@ngrt.de>  (preferred)

Fidonet: 2:2490/2575

EPILOGUE

Suggestions and bug-reports are always welcome. Well ... bug-reports

are perhaps not *that* welcome ... ;)