LINUX Installation

This installation guide describes the procedures to install NetMos Technology Nm9735CV, Nm9745CV, Nm9835CV, Nm9845CV PCI serial ports in Linux platform.

Linux Platform

l Operating System		: RedHat, SuSe

l Terminal Emulation AP	: minicom

l Internet Dialer		: Kppp

Installation Serial Ports

Find the available serial ports

Since Linux only support 4 serial ports (ttyS0, ttyS1, ttyS2, ttyS3) under the default condition. Most likely, ttyS0 & ttyS1 are supported by mother board's built-in serial controllers and ttyS2 & ttyS3 are free for additional I/O card.

Serial ports could be checked by the following commands.

setserial  /dev/ttyS0 -a 		(COM-1)

setserial  /dev/ttyS1 -a 		(COM-2)

setserial  /dev/ttyS2 -a 		(COM-3)

setserial  /dev/ttyS3 -a 		(COM-4)

If COM-1 is used by mouse, the response is similar to

/dev/ttyS0 : Device or resource busy

If the COM-1 does not attach any device, the response is similar to

/dev/ttyS0, Line 0, 	UART: 16550A, Port: 0x3f8, irq: 4

Baud_base: 115200, clos_delay: 50, divisor: 0

closing_wait: 3000, closing_wait2: infinite

Flags: spd_normal skip_test

In case ttyS2 (COM-3) is free, the response for command  setserial /dev/ttyS2 -a is shown below.

/dev/ttyS2, Line 2, 	UART: unknown, Port: 0x3e8, irq: 4

Baud_base: 115200, clos_delay: 50, divisor: 0

closing_wait: 3000, closing_wait2: infinite

Flags: spd_normal skip_test

Find the PCI card resource ( I/O port address & IRQ) for the serial ports

Enter the command  more  /proc/pci

The response is similar to the following

Bus 0, Device 11, function 0:

Serial controller : Unknown vendor Unknown device (rev 01).

Vendor id=9710, Device id=9735

Device id=9745

Device id=9835

Device id=9845

Medium devsel. Fast back-to-back capable. IRQ 11

Board with Nm9735CV part

I/O at 0xc000 [0xc001]	serial port 1

0xc008 [0xc009]	serial port 2

Board with Nm9745CV part

I/O at 0xc000 [0xc001]	serial port 1

0xc008 [0xc009]	serial port 2

0xc010 [0xc011]	device 1

0xc018 [0xc019]	device 2

0xc020 [0xc021]	device 3

0xc028 [0xc029]	device 4

Board with Nm9835CV part

I/O at 0xc000 [0xc001]	serial port 1

I/O at 0xc400 [0xc401]	serial port 2

I/O at 0xc800 [0xc801]	not used

I/O at 0xd000 [0xd001]	not used

I/O at 0xd400 [0xd401]	not used

I/O at 0xd800 [0xd801]	not used

Board with Nm9845CV part

I/O at 0xc000 [0xc001]	serial port 1

I/O at 0xc400 [0xc401]	serial port 2

I/O at 0xc800 [0xc801]	device 1

I/O at 0xd000 [0xd001]	device 2

I/O at 0xd400 [0xd401]	device 3

I/O at 0xd800 [0xd801]	device 4

Board with Nm9755CV part

I/O at 0xc000 [0xc001]	not used

I/O at 0xc400 [0xc401]	not used

I/O at 0xc800 [0xc801]	device 1

I/O at 0xd000 [0xd001]	device 2

I/O at 0xd400 [0xd401]	device 3

I/O at 0xd800 [0xd801]	device 4

From the  /proc/pci file, it is possible to find the PCI card's I/O port address and IRQ. Especially, the NetMos parts always shows

Vendor id=9710, Device id=9735

Device id=9745

Device id=9835

Device id=9845

Configure the parameters for ttyS2 or ttyS3

Enter (if ttyS2 or ttyS3 are free)

setserial  /dev/ttyS2 port 0xc000 UART 16550A

irq 11 Baud_base 115200

setserial  /dev/ttyS3 port 0xc008 UART 16550A

irq 11 Baud_base 115200

Check the setting for ttyS2 or ttyS3

Type setserial  /dev/ttyS2 -a

/dev/ttyS2, Line 2, UART: 16550A, Port: 0xc000, irq: 11

Baud_base: 115200, clos_delay: 50, divisor: 0

closing_wait: 3000, closing_wait2: infinite

Flags: spd_normal skip_test

The ttyS2 or ttyS3 are ready for application

Installation Parallel Ports

Installation Guide for PCI Parallel Ports

This installation guide describes the procedure to install the Nm9705,Nm9735CV, Nm9805CV, Nm9715CV, Nm9835CV, Nm9755CV PCI parallel ports in Linux platform.

Linux kernel provide a 'parport' code to support parallel port (/dev/lp0,  /dev/lp1, /dev/lp2). This code provides the ability to share one port between multiple devices. And it is loadable when kernel is running.

The 'parport' code is split into two parts : generic (which deals with port sharing) and architecture-dependent eg. X86, SPARC. (which deals with actually using the port). Thus, please take the following steps to install 'parport' module!

Check the on-board parallel port's hardware resources

Most likely each motherboard has a built-in parallel port. And its hardware resources are settable with BIOS utility.

Enter the command  more  /proc/pci

The response is similar to the following

Bus 0, Device 11, function 0:

Serial controller : Unknown vendor Unknown device (rev 01).

Vendor id=9710, Device id=9705

Device id=9735

Device id=9805

Device id=9815

Device id=9835

Device id=9855

Medium devsel. Fast back-to-back capable. IRQ 11

Board with Nm9705CV part

I/O at 0xc000 [0xc001]	printer port

0xc008 [0xc009]	ECP/EPP config registers

Board with Nm9735CV part

I/O at 0xc000 [0xc001]	not used

0xc008 [0xc009]	not used

0xc010 [0xc011]	printer port

0xc018 [0xc019]	ECP/EPP config registers

Board with Nm9805CV part

I/O at 0xc000 [0xc001]	printer port

I/O at 0xc400 [0xc401]	ECP/EPP config registers

I/O at 0xc800 [0xc801]	not used

I/O at 0xd000 [0xd001]	not used

I/O at 0xd400 [0xd401]	not used

I/O at 0xd800 [0xd801]	not used

Board with Nm9715CV part

I/O at 0xc000 [0xc001]	printer port 1

I/O at 0xc400 [0xc401]	ECP/EPP config registers 1

I/O at 0xc800 [0xc801]	printer port 2

I/O at 0xd000 [0xd001]	ECP/EPP config registers 2

I/O at 0xd400 [0xd401]	not used

I/O at 0xd800 [0xd801]	not used

Board with Nm9835CV part

I/O at 0xc000 [0xc001]	not used

I/O at 0xc400 [0xc401]	not used

I/O at 0xc800 [0xc801]	printer port

I/O at 0xd000 [0xd001]	ECP/EPP config registers

I/O at 0xd400 [0xd401]	not used

I/O at 0xd800 [0xd801]	not used

Board with Nm9755CV part

I/O at 0xc000 [0xc001]	printer port 1

I/O at 0xc400 [0xc401]	ECP/EPP config registers 1

I/O at 0xc800 [0xc801]	printer port 2

I/O at 0xd000 [0xd001]	ECP/EPP config registers 2

I/O at 0xd400 [0xd401]	not used

I/O at 0xd800 [0xd801]	not used

Install 'parport' module

To load the generic 'parport' code

Type 			insmod parport.o

To load the architecture-dependent code to tell 'parport' code

insmod parport_pc.o  io=0x378,0xc000,0xc800

irq=7, irq=11, none

You have three PC-style ports, one at 0x378 with IRQ 7, one at

0xc000 with  IRQ11, one at 0xc800 with no IRQ.

Check the attached printer device [optional]

Once the architecture-dependent part of the paraport code is loaded into the kernel. You could enter the following command

insmod parport_probe.o

To check any attached devices and log a message similar to

parport0: Printer, epson

Then 			/dev/lp0, /dev/lp1 and /dev/lp2 are ready for service