FFFFFFF  RRRRRR     EEEEEEE   EEEEEEE

F        R     R    E         E

F        R     R    E         E         FREE

FFFFF    RRRRRR     EEEEE     EEEEE

F        R   R      E         E         FORM

F        R    R     E         E

F        R     R    EEEEEEE   EEEEEEE

The Ultimate Floppy Disk Formatter

(version 2.3)

94-Nov-01

======================================

INSTALLATION AND USAGE GUIDE

======================================

Copyright (c) 1988-1994

Herne Data Systems Ltd.

P.O. Box 250

Tiverton, ON  N0G 2T0

CANADA

voice or FAX (519)366-2732

CompuServe 72060,1153

Internet herne@wbb.com

(FREE.DOC v2.3 94-11-01)

TABLE OF CONTENTS

THE FINE PRINT  . . . . . . . . . . . . . . . . . . . . . . .   1

DISCLAIMER - AGREEMENT   . . . . . . . . . . . . . . . .   1

REGISTRATION BENEFITS  . . . . . . . . . . . . . . . . .   2

REGISTRATION AND CUSTOMER SUPPORT ADDRESS  . . . . . . .   2

ASP OMBUDSMAN  . . . . . . . . . . . . . . . . . . . . .   3

DEFINITION OF SHAREWARE  . . . . . . . . . . . . . . . .   3

INTRODUCTION  . . . . . . . . . . . . . . . . . . . . . . . .   4

A Note on Terminology  . . . . . . . . . . . . . . . . .   4

Making a Backup Copy . . . . . . . . . . . . . . . . . .   5

Upgrading From an Earlier Version  . . . . . . . . . . .   6

FREEFORMING . . . . . . . . . . . . . . . . . . . . . . . . .   7

Selecting an Entry . . . . . . . . . . . . . . . . . . .   7

Formatting a DOS Disk  . . . . . . . . . . . . . . . . .   8

Formatting a Non-DOS Disk  . . . . . . . . . . . . . . .  10

MODIFYING DISK PARAMETERS . . . . . . . . . . . . . . . . . .  10

General Data Area  . . . . . . . . . . . . . . . . . . .  11

Zone Specific Data . . . . . . . . . . . . . . . . . . .  12

Using Multiple Zones . . . . . . . . . . . . . . . . . .  14

Saving the Parameters  . . . . . . . . . . . . . . . . .  16

SETTING DEFAULTS  . . . . . . . . . . . . . . . . . . . . . .  16

EXITING FREEFORM  . . . . . . . . . . . . . . . . . . . . . .  16

USING SMAX.COM  . . . . . . . . . . . . . . . . . . . . . . .  17

When to Use SMAX . . . . . . . . . . . . . . . . . . . .  17

Installing SMAX  . . . . . . . . . . . . . . . . . . . .  18

Disabling SMAX . . . . . . . . . . . . . . . . . . . . .  20

80 Cylinder Double Density Mode  . . . . . . . . . . . .  21

THE TECHNICAL DETAILS . . . . . . . . . . . . . . . . . . . .  22

Technical Data of MAXI Style Disks . . . . . . . . . . .  22

APPENDIX A: ENHANCEMENTS AND IMPROVEMENTS . . . . . . . . . .  23

New Features of v2.3 . . . . . . . . . . . . . . . . . .  23

i

FreeForm v2.3                                        Users' Guide

-----------------------------------------------------------------

THE FINE PRINT

DISCLAIMER - AGREEMENT

Users of FreeForm must accept this disclaimer of warranty:

"FreeForm is supplied as is.  The author disclaims all

warranties, expressed or implied, including, without

limitation, the warranties of merchantability and of fitness

for any purpose. The author assumes no liability for

damages, direct or consequential, which may result from the

use of FreeForm.  FreeForm is intended to format blank disks

only.  Because it will destroy all existing data on a floppy

disk, Herne Data Systems assumes no liability for losses

caused by misuse or abuse of this product.   This

responsibility rests solely with the end user."

Please note that FreeForm relies on quirks in the MS-DOS/PC-DOS

operating system and BIOS ROM's. Although it has been tested on a

wide variety of systems with MS/PC-DOS versions 3.2x to 6.1 and

DR-DOS 5.0 and later,  it cannot be guaranteed to work with all

systems, especially ones which have been heavily customized and

modified (including the installation of non-standard disk drives

and controller cards) by the user; nor can it be guaranteed to

work with future releases of MS/PC-DOS.

FreeForm is a "shareware program" and is provided at no charge to

the user for evaluation.  Feel free to share it with your

friends, but please do not give it away altered or as part of

another system.  The essence of "user-supported" software is to

provide personal computer users with quality software without

high prices, and yet to provide incentive for programmers to

continue to develop new products.  If you find this program

useful and find that you continue to use FreeForm after a 30 day

trial period, you must make a registration payment of $25 (US) or

$29 (CDN) (plus $2 postage) to Herne Data Systems Ltd.  Cheque,

money order, VISA credit card and corporate/institutional

purchase orders are accepted.  The single user registration fee

will license one copy for use on any one computer at any one

time.  You must treat this software just like a book.  An example

is that this software may be used by any number of people and may

be freely moved from one computer location to another, so long as

there is no possibility of it being used at one location while

it's being used at another. Just as a book cannot be read by two

different persons at the same time.

Commercial, corporate and institutional users of FreeForm must

register and pay for their copies of FreeForm within 30 days of

first use or their license is withdrawn.  Site Licenses are

1

FreeForm v2.3                                        Users' Guide

-----------------------------------------------------------------

available for $100 (US) or $125 (CDN) directly from Herne Data

Systems Ltd.

Any individual, corporation or institution distributing FreeForm

for any kind of remuneration must first contact Herne Data

Systems Ltd. at the address below for authorization.  This

authorization will be granted automatically to distributors

recognized by the ASP as adhering to its guidelines for shareware

distributors, and such distributors may begin offering FreeForm

immediately (However Herne Data Systems Ltd. must still be

advised so that the distributor can be kept up-to-date with the

latest version of FreeForm.).

You are encouraged to pass a copy of FreeForm along to your

friends for evaluation.  Please encourage them to register their

copy if they find that they can use it.

REGISTRATION BENEFITS

All registered users will receive a copy of the latest version of

FreeForm, along with free technical support and updates for 60

days.  Registered users can also update to later versions (beyond

the initial 60 day free support period) for a nominal update fee

of $5.00 plus $2.00 postage and handling.

REGISTRATION AND CUSTOMER SUPPORT ADDRESS

If you have already registered, THANK YOU!  If you have not

registered, please consider doing so.  Simply fill out the

registration form in the REGISTER.DOC file and return it to the

address below with your check or money order for $27 (US) or $31

(CDN) - (includes registration plus $2 postage for US or Canada).

If outside US or Canada, please add an additional $2 postage

(total of $29 (US) or $33 (CDN)).

Please address all registration requests and customer support

inquiries to:

Herne Data Systems Ltd.,

PO Box 250, Tiverton, ON  N0G 2T0  CANADA

voice or FAX   (519)366-2732

Technical support can also be obtained on CompuServe by directing

inquiries to 72060,1153; or in the Internet to herne@wbb.com.

The extent of technical support for non-registered users is

limited and is solely at the discretion of Herne Data Systems Ltd.

2

FreeForm v2.3                                        Users' Guide

-----------------------------------------------------------------

ASP OMBUDSMAN

This program is produced by a member of the Association of

Shareware Professionals (ASP).  ASP wants to make sure that the

shareware principle works for you.  If you are unable to resolve

a shareware-related problem with an ASP member by contacting the

member directly, ASP may be able to help.  The ASP Ombudsman can

help you resolve a dispute or problem with an ASP member, but

does not provide technical support for members' products.  Please

write to the ASP Ombudsman at: 545 Grover Road, Muskegon, MI

49442  USA  or send a Compuserve message via easyplex to ASP

Ombudsman 70007,3536

DEFINITION OF SHAREWARE

Shareware distribution gives users a chance to try software

before buying it. If you try a Shareware program and continue

using it after a reasonable evaluation period, you are expected

to register. Individual programs differ on details -- some

request registration while others require it, some specify a

maximum trial period. With registration, you get anything from

the simple right to continue using the software to an updated

program with printed manual.

Copyright laws apply to both Shareware and commercial software,

and the copyright holder retains all rights, with a few specific

exceptions as stated below. Shareware authors are accomplished

programmers, just like commercial authors, and the programs are

of comparable quality. (In both cases, there are good programs

and bad ones!) The main difference is in the method of

distribution. The author specifically grants the right to copy

and distribute the software, either to all and sundry or to a

specific group. For example, some authors require written

permission before a commercial disk vendor may copy their

Shareware.

Shareware is a distribution method, not a type of software. You

should find software that suits your needs and pocketbook,

whether it's commercial or Shareware. The Shareware system makes

fitting your needs easier, because you can try before you buy.

And because the overhead is low, prices are low also. Shareware

has the ultimate money-back guarantee -- if you don't use the

product, you don't pay for it.

3

FreeForm v2.3                                        Users' Guide

-----------------------------------------------------------------

INTRODUCTION

FreeForm is a menu driven universal floppy disk formatting

utility. Unlike the MS-DOS FORMAT.COM program, FreeForm allows

you to format virtually any kind of floppy disk (including

non-MS-DOS types) with an MS-DOS compatible computer (including

the PS/2 line).  All styles of floppy drives are supported,

including the external drives and secondary floppy controller

cards used with some machines.  FreeForm will not let you format

a hard disk.  In order to use FreeForm, you will need DOS version

3.20 or later (or DR-DOS 5.0 or later) and at least 256k bytes of

RAM.

Although it is easy to use by people of all levels of programming

experience and provides an simple method for formatting various

kinds of standard DOS floppy disks, the advanced features of

FreeForm are intended primarily for intermediate to advanced

MS-DOS users who will appreciate the complete flexibility that it

provides.  With flexibility, however, comes some degree of

responsibility.  FreeForm does a minimum of error checking on the

input parameters.  It will accept a wide range of non-standard

responses and attempt to perform them (not always successfully,

mind you).  In short, be aware of the implications of what you

are trying to do.

Non-MS-DOS disks and special MS-DOS disks can be formatted in up

to three different "density zones" on the same disk.  That is,

different areas of the disk can be formatted with different

numbers of sectors, different sector sizes or numbering schemes.

Using this feature, you can create non-standard areas on a DOS

disk (such as a track full of 256 byte sectors) for hidden data

or copy protection purposes, for example.  The user also has full

control over disk parameters such as the hard skew (sector

interleave) and blank sector fill byte for each zone.

Up to 100 sets of parameters can be stored in a parameter file

for later recall and use from the main menu of FreeForm.

A Note on Terminology

In various parts of this documentation you will be asked to

'press a key', 'type in' or 'enter a value'.  These phrases have

slightly different  meanings as outlined below.

'Press a key' refers to merely pressing the key down briefly the

releasing it again.  If you are asked to press a <Control> key,

4

FreeForm v2.3                                        Users' Guide

-----------------------------------------------------------------

then hold down the key marked <Ctrl> or <Control> and press the

requested alphanumeric key at the same time (just as you would

when using the <Shift> key).

'Type in' and 'enter a value' refer to the process of pressing

one or more alphanumeric keys in a given sequence to form a

string of characters (just as if you were typing on the keyboard)

followed by pressing the key marked <Enter> (or <Return> on some

keyboards).  Pressing the <Enter> key is a signal that you have

finished your keyboard input and the program can resume.

<cursor up>, <cursor down>, <cursor left> and <cursor right>

refer to the cursor control keys, located either in a separate

cursor control keypad, or as part of the numeric keypad,

depending on the layout of your keyboard.  These keys are used to

scroll list entries and/or to move the cursor on the full screen

editor in the parameter entry/edit mode.  The keys are normally

marked with arrows on the key caps corresponding to the movement

direction.

<Home>, <End>, <PgUp> and <PgDn> keys are generally located with

or near the cursor control keys outlined above.  Note that some

laptop computers with compact keyboard arrangements may require

special multiple key combinations to access these keys.  <Home>,

<PgUp> and <PgDn> are used with the main menu to scroll through

the selection list, while <End> is used with the parameter

entry/edit screen to return to the main menu screen.  The keys

are generally marked with the above words.

<Insert>, <Delete> and <Backspace> are screen editing keys.  With

the FreeForm parameter entry/edit screen, these keys have their

normal functions.

Making a Backup Copy

Before using FreeForm for the first time, it is a good idea to

make a backup copy of the program disk for everyday use.  Since

the FreeForm disk is not copy protected, you can copy the files

on the disk using the normal DOS COPY command.  The FreeForm

distribution disk will contain the following files:

FREE.EXE       The main FreeForm program file

FREE23.DEF     The FreeForm data file containing disk type

definitions.

FREE.DOC       This documentation file.

5

FreeForm v2.3                                        Users' Guide

-----------------------------------------------------------------

SMAX.COM       The TSR driver to use extended capacity DOS disks

on some systems.

UPDATE.DOC     Summary of latest features not covered in this

manual.

REGISTER.DOC   Shareware registration form.

With a two drive system, put the FreeForm disk in drive A: and

the formatted backup disk in drive B:.  Next type in:

COPY  A:*.*  B:

After a few moments, the copy process will be complete.  Store

your original FreeForm disk in a safe place and use the backup

copy.

With a hard disk system, you can first create a special

subdirectory for the FreeForm files, such as:

MD C:\FREEFORM

Next, change to this subdirectory:

CD\FREEFORM

Then insert the FreeForm disk in drive A: and type in:

COPY  A:*.*  C:

The DOS COPY command can be used for other drives or system

configurations by using the appropriate target drive and path

specification.

Upgrading From an Earlier Version

The disk definition file for version 2.3 is different than

previous versions.  To mark the difference, it has been re-named

to FREE23.DEF (previously called FREE.DEF).  If you are upgrading

from an earlier version of FreeForm and have a number of custom

defined disk formats in your old FREE.DEF, you can import these

directly into the new file with the following method using DOS

(or your favourite file manager program):

RENAME FREE23.DEF  FREE23.NEW

COPY FREE.DEF FREE.OLD

6

FreeForm v2.3                                        Users' Guide

-----------------------------------------------------------------

(this will keep a backup copy of the original new FREE23.DEF file

and the old FREE.DEF file).

RENAME FREE.DEF FREE23.DEF

Then run FreeForm normally.  The program will recognize the old

file format and automatically convert it to the new format.

However, the converted file will not contain the new DOS formats

supported by version 2.3:  1.7 meg on a 1.44 meg drive, 2.8 meg

and 3.2 meg on a 2.8 meg drive.  If you require these formats,

you must add them to the new file manually (see MODIFYING DISK

PARAMETERS, below).

FREEFORMING

Once you have copied the files to your directory, you are ready

to use FreeForm.  At the DOS prompt, type in:

FREE

After a few moments the sign-on screen will appear as the data

tables are being loaded from the file FREE23.DEF.  When the

process is complete, you will be taken to the main menu screen.

(The time required to load the FREE23.DEF file depends on the

speed of your disk system and the number of parameter sets you

have stored in it.  Only those parameter sets which you have

actually defined will be saved in the FREE23.DEF file.)

Selecting an Entry

The main menu screen is divided up into several portions.  At the

left edge, a boxed window displays a ten entry portion of the

list of disk types available in your parameter file.  To select

an entry, use the <cursor up> and <cursor down> arrow keys on

your computer's cursor control keypad.   To scroll the list

faster, you can use the <PgDn> (scroll ahead) and <PgUp> (scroll

back) keys to move one page (ten entries) at a time.  The <Home>

key will take you back to the first entry in the list.

The currently selected entry will be displayed in reverse video.

If you wish to display or edit the parameter settings associated

with this particular disk type, press the <Enter> key.  This will

take you to the parameter entry/edit screen, described below.

The FREE23.DEF file supplied on the distribution disk contains

7

FreeForm v2.3                                        Users' Guide

-----------------------------------------------------------------

the parameters for all of the DOS disk types as well as a

selection of some of the more popular CP/M types.  You can add

more disk types at any time (up to 100 sets total can be stored

in the file) by going past the last one in the current file and

entering the new values.  (New entries are initially assigned

default values which can be edited to suit your needs.)

To change the disk drive to perform the formatting on, use the

<cursor left> and <cursor right> arrow keys.  The currently

selected drive is displayed in a window in the upper right

portion of the screen.  Note that only drive letters associated

with valid physical floppy disk drives (as determined during the

initialization process) actually can be selected.

Formatting a DOS Disk

FreeForm supports a total of 14 types of standard DOS and DOS

compatible extended capacity disk types (MAXI format disks).  The

supported DOS disk types are:

Type Bytes     Physical Layout               Size

D1   160 k     SS, 8 sector, 40 track        5.25 inch

D2   180 k     SS, 9 sector, 40 track        5.25 inch

D3   320 k     DS, 8 sector, 40 track        5.25 inch

D4   360 k     DS, 9 sector, 40 track        5.25 inch

D5   1.2 meg   DS, 15 sector, 80 track       5.25 inch

D6   720 k     DS, 9 sector, 80 track        3.5 inch

D7   1.44 meg  DS, 18 sector, 80 track       3.5 inch

D8   2.8 meg   DS, 36 sector, 80 track       3.5 inch

Types D1 to D3 are more or less obsolete.  D4 is the standard PC

and XT type.  D5 is a high density AT type.  D6 is a 3-1/2 inch

type used with some older laptop computers.  D7 is a standard

3-1/2 inch high density type.  D8 is used with the new extra high

density drives. (Note that D6 can also be used on a 5-1/4 inch AT

drive to produce DOS compatible 720k 5-1/4 disks, and D5 can be

used on the 1.44 meg drives.)

In addition to these standard DOS disk types, six new DOS

compatible formats have been included for users of DOS versions

3.2 and later.  These are listed in the next table.

Type Bytes     Physical Layout               Size

D9   420k      DS, 10 sector, 42 track       5.25 inch

8

FreeForm v2.3                                        Users' Guide

-----------------------------------------------------------------

D10  800k      DS, 10 sector, 82 track       3.5 inch

D11  1.4 meg   DS, 17 sector, 82 track       5.25 inch

D12  1.6 meg   DS, 20 sector, 82 track       3.5 inch

D13  1.7 meg   DS, 21 sector, 82 track       3.5 inch

D14  3.2 meg   DS, 39 sector, 82 track       3.5 inch

After selecting a disk type on the main menu screen, you are

ready to begin formatting a new disk.  There are two ways to

format the disk.  The first method is the 'Fast Format' which

merely sends the format command to the drive and does not bother

to attempt to verify that the new disk has been properly

formatted.  This method, which is much faster than the 'Format

with Verify' option, is accessed by pressing the <F1> key.   (In

fact, it does not even bother to check that a disk is in the

drive before trying to format.)

The second method is the 'Format with Verify' option selected by

pressing the <F2> key.  This option will attempt to read each

sector after a track has been formatted.  If an error is

detected, it will be recorded internally, but the process will

continue.  This 'non-fatal' error reporting method has been used

to allow you to put deliberate errors on the disk if you so wish.

After selecting either formatting method, you will be prompted to

insert a new disk in the specified drive and then press <Enter>

to continue or <Esc> to abort the operation.  Once formatting has

begun, the status window at the bottom of the screen will display

the current zone, cylinder (track) and side being formatted.

At the end of the formatting cycle, you will be asked if you wish

to include the system files for DOS disks.  Type in Y for YES if

you wish to make the disk 'boot able' or any other response to

create a non-system or data only disk.  Pressing <Enter> alone

has the same effect as a N for NO response.

The three system files IBMBIO.COM (or IO.SYS), IBMDOS.COM (or

MSDOS.SYS) and COMMAND.COM are required to create a system 'boot'

disk.  These three files will be copied from the system path

specified during on the setup screen (see below).  (The default

is C:\ - the root directory of your hard drive).  If the files

are not found in this DOS path, you will be prompted to enter a

new path.

After copying the system files, you will be given a chance to

enter a volume label for the disk.  Enter a text string of 1 to

11 characters for the label, or just press <Enter> alone if you

do not wish to include the label entry.

9

FreeForm v2.3                                        Users' Guide

-----------------------------------------------------------------

After formatting has been completed, you will be asked if you

wish to format another disk of the same type.  Enter Y if you do

or any other response if you do not.  If the 'format with verify'

was selected, a message indicating the total number of errors

detected will also be displayed at this time.  After all

formatting has been completed, you will be returned to the main

menu screen.

Formatting a Non-DOS Disk

The procedure for formatting a non-DOS disk is virtually

identical to that for formatting a DOS disk.  Select a non-DOS

disk type from the main menu, then press one of the formatting

function keys.  Both Fast Format (<F1>) and Format with Verify

(<F2>) are supported for non-DOS disks.

With a non-DOS disk, you will not be asked if you wish to include

system files or a volume label when the disk has been formatted.

MODIFYING DISK PARAMETERS

The edit screen displays all of the user changeable options for

the disk format along with the values for the currently selected

parameter set.   To reach the edit screen, select an entry on the

main menu screen, then press <enter>.  This will take you

directly to the edit screen for that particular entry.  Once you

are on the edit screen you can page between entries by using the

<PgUp> and <PgDn> keys.

The parameter to edit is selected using the <cursor up> and

<cursor down> keys to scroll through the list of options.  The

currently selected option is highlighted in reverse video.

(Note: if you have incorrectly specified colors for a monochrome

display adaptor using the SETUP option, you may not get reverse

video.)  The parameter value can then be edited using the

alpha-numeric keys.  Cursor movement keys, as well as <Insert>,

<Delete> and <Back Space> can all be used in the editing process.

To delete the currently displayed parameter set, press

<Control>-D.  You will be asked to confirm such a deletion

because once done, it cannot be recovered (unless you exit

FreeForm without saving the new parameter file).  To re-sort the

entries in you file, press <Control>-S.  The entries will be

sorted by disk type (all DOS types will come to the top of the

list) and name (alphabetically).

10

FreeForm v2.3                                        Users' Guide

-----------------------------------------------------------------

To save the edited parameter set, press the <F1> function key.

To return to the main menu without saving, press the escape

(<Esc>) key.  (If you do not save the new parameters at this

point, the modified parameters will still be available for use

during the current session.  However, if you do not save them

before you exit FreeForm, the changes will be lost the next time

you start FreeForm).

For each option, a brief description appears in the box at the

bottom of the screen.  Use these as a guide for entering values.

All numeric parameters are entered as decimal values EXCEPT the

'Fill Byte' parameter which is entered as a hexadecimal value.

Although some error checking is done at this point, not all

combinations can be cross checked.  If FreeForm comes across a

value that is definitely outside of the physical limitations of

your equipment (such as trying to format an 80 track disk on a 40

track drive) it will beep at you and display an error message.

General Data Area

The first option which can be edited is the disk name.  This is

used for the main menu display but is not otherwise part of the

formatting process.  Any ASCII string of up to 16 characters can

be used.

The next option is the disk type.  This can be N for non-MS-DOS

or D1 to D14 for DOS disks. The disk formats corresponding to

each DOS disk type are outlined above under 'Formatting a DOS

Disk'.   The extended disk types (D9 to D14) will maximize the

space available to the user by adding extra sectors and tracks.

The disks are fully supported and are even bootable by some (but

not all) combinations of DOS and hardware.  Note that some

combinations of DOS and hardware require the use of the  memory

resident 'driver' program SMAX.COM, outlined below.  (It is

generally BIOS dependent).

If a DOS type disk is selected, three extra parameters can be

changed:  number of root directory entries (must be a multiple of

16), number of copies of the FAT stored on the disk (1 or

normally 2), allocation unit size (normally 1 for high density

disks or 2 for double density, can be 1, 2, 4, or 8).  A few

extra kb of space can be squeezed out of a disk by:  decreasing

the number of entries in the root directory (each 16 entries

takes up 512 bytes); using one copy of the FAT (DOS keeps 2

identical copies, but normally only uses one.  This can add up to

5 k bytes of space); increasing the allocation unit size (this

determines how many sectors are used for the FAT).  All of these

11

FreeForm v2.3                                        Users' Guide

-----------------------------------------------------------------

measures are most effective if the disk is to be used to store

only a few very large files, such as archives (ZIP files).  Use

the <Tab> key to move from one field to the next.

The disk capacity of all DOS disks may be further extended by

increasing the number of formatted tracks.  The 42 and 82 tracks

used by the 5-1/4 versions and the 82 tracks used by the 3-1/2

versions represent the maximum span of most disk drives.

However, certain individual drives might be able to go beyond

even this range.  If you are will to experiment with your drives,

you will gain an extra 10k to 40k of user space for each extra

track you can squeeze.  The extra track versions of D1 to D8

standard DOS disks can be used transparently with all versions of

DOS from 2.1 onwards.

If you have specified a DOS type disk, the rest of the parameters

in ZONE 1 will be re-set to the default values corresponding to

that disk type.  However, they can be re-edited if you wish

(beware of the consequences however).  DOS disks will include the

appropriate BOOT record, directory and FAT areas.  Non DOS disk

will not have any special areas set up.

The 'Sector Density' corresponds to the disk density: D for

double density (normally 360k or 720k), H for high density (1.2

meg or 1.44 meg), or E for extra high density (2.8 meg).   This

allows you to create double density (DD) disks in high density

(HD) or extra high density (ED) drives and HD disks in ED drives.

The 'Track Density' is used to determine what kind of disk will

be formatted in an 80 track drive.  Set the value to 40 if you

wish to format a 40 track disk in an 80 track drive.  For a 40

track drive, this must be set to 40.  Note that 40 track disks

formatted on 80 track drives are often unreliable for reading and

writing in 40 track drives.  They are usually fine for other 80

track drives, however.  (The head on the 80 track drive is only

half as wide as the 40 track.  Therefore the tracks are only

'half written' which may cause errors when trying to read/write

with a wider 40 track head.)

Zone Specific Data

The remainder of the parameters can take on separate different

values for up to three different areas of the disk.  The disk

area or 'zone' is selected on the editing screen by pressing the

<Tab> key.

The 'Sides' parameter determines the physical side of the disk

12

FreeForm v2.3                                        Users' Guide

-----------------------------------------------------------------

associated with this zone.  It can have values of DS for double

sided (i.e. both sides of the disk are the same), S0 if the

parameters only apply to head 0 (i.e. a normal single sided disk)

or S1 for a single sided zone on the back of a normal single

sided disk.  If the given zone is not used, then this field

should be left blank.

The 'Sector Size' can have a value of 256, 512, or 1024.  DOS

uses a size of 512 for floppy disks, while non-DOS disks can use

any of the values.  Each zone can have its own sector size.

'Sectors per track' is used in conjunction with the sector size

to determine the capacity of the disk.  The maximum number of

sectors is determined by the physical limits of the drive.  As a

general rule this is:

Drive type               Sector Size

256         512            1024

360 k             18          10             5

or 720k

1.2 meg           26          17             8

1.44 meg          30          21             10

2.8 meg           60          39             20

'First sector number' determines the sector numbering scheme.

DOS disk always have a first sector number of 1.  Other non-DOS

disks can have sectors starting at 0 or any number up to 255 in

special cases.  An example of this would be KayPro IV CP/M disks

which have sectors numbered from 0 to 9 on side 0 and 10 to 19 on

side 2.  With FreeForm a disk of this type is created by

specifying zone 1 as S0 and zone 2 as S1, each with the

appropriate sector size and numbering.

'Sector Skew Factor' determines how the sectors are placed on the

disk.  A skew factor of 0 indicates that sectors are numerically

adjacent.  That is, sectors are in the order of 1, 2, 3, 4, etc.

A value of 1 indicates that 1 sector is skipped between

numerically consecutive sectors.  For example, the sectors might

13

FreeForm v2.3                                        Users' Guide

-----------------------------------------------------------------

be in the order of 1, 5, 2, 6, 3, etc. in this case.  The valid

skews for a given number of sectors is a complex relationship

involving mathematical common factors.  In general, the value of

(skew + 1) cannot divide evenly into the number of sectors.  For

example, you cannot have 16 sectors with a skew of 1 or 3

(16/(1+1)=8 and 16/(3+1)=4), or 9 sectors with a skew of 2

(9/(2+1)=3).  In addition, the (skew + 1) and number of sectors

cannot have any common factors.  For example, you cannot have 9

sectors with a skew of 5 (both 5+1 and 9 are evenly divisible by

3) or 10 sectors with a skew of 3 (both 3+1 and 10 are evenly

divisible by 2).  If all this sounds confusing, don't worry,

FreeForm, will let you know if it thinks the value is invalid.

Generally, you can use a skew of 0 or 1 with an odd number of

sectors (e.g. 5, 9, 15, etc); or 0, 2 or 4 with an even number of

sectors (e.g. 8, 16, etc).

'First Track' tells FreeForm where to start formatting the disk

for the selected zone.  To start at the beginning, first track

should be set to 0.  Starting elsewhere indicates that some of

the disk surface will be skipped prior to the start of the

formatting operation.  Used in conjunction with the 'Last Track'

option, this allows you to selectively format given areas of a

disk in different configurations.

'Last Track' tells FreeForm where to finish formatting the disk

for the selected zone.  It can have a value of greater than or

equal to 'First Track' of the same zone.  The value must also be

less than 'Track Density' + 5.  This allows you to go a bit

beyond the normal capacity of the drive to add extra tracks where

desired.  Note that most drives will only go one or two tracks

beyond the normal limit.  Attempting to go further may cause the

drive to jam up.

The final user selectable parameter is the 'Fill Byte'.  This is

the value written to the empty sectors as they are created on the

disk.  For DOS disks, fill byte has a value of hex F6.  For CP/M

disks, it has a value of hex E5.  Note that this is the only

field where numbers are entered as hex values instead of decimal

values.

Using Multiple Zones

FreeForm uses a multi zone disk formatting concept.  Each disk

can be divided into up to three distinct areas.  Each zone can be

thought of as a separate area of the disk with individual

properties.   The zones can be used to create disks which have

different characteristics in different areas of the disk or to

14

FreeForm v2.3                                        Users' Guide

-----------------------------------------------------------------

create special custom disk formats for data security copy

protection, etc..  The zones can be sequential or they can

overlap or they can be broken.

An example of a sequentially zoned disk can be found with the

Epson QX-10 CP/M disk format.  With this type, zone 1 is

comprised of tracks 0 to 3.  In this region of the disk, the

tracks on both sides of the disk are formatted with 16 sectors

per track, 256 bytes per sector.  In zone 2, which comprises the

remainder of the disk (tracks 4 to 39), the tracks on both sides

of the disk are formatted with 10 sectors per track, 512 bytes

per sector.

An example of an overlapping zoned disk can be created by adding

a track of non-standard sectors to, for example, a standard DOS

disk.  For example, you can start with disk type D4 to create a

normal 360k DOS disk complete with boot record, FAT and directory

(even the system files if you so choose).  Next move to zone 2 on

the edit screen by using the <cursor down> key to highlight the

Sides parameter, then pressing the <Tab> key to reach zone 2.

Next enter some parameters, say S1 for the Sides parameter, 256

for the sector size parameter, 16 for the number of sectors per

track, 32 for the first sector number, 0 for the skew factor, 39

for the first track, 39 for the last track, and 27 for the fill

byte.  This strange concoction will create a disk that will, for

all intents and purposes be almost fully DOS compatible, except

that will have a non-standard track on the back side.  Track 39

on side 1 will have 16 sectors of 256 bytes each, numbered 32 to

47, filled with the value of hex 27.  All you need to do to

create a copy protected program is to have your program check one

or more sectors on this track (using the BIOS disk interrupt

because you cannot read this track using standard DOS calls) and

check for the required data pattern.  If an error occurs when you

try to read 256 byte sectors on this track, then the disk is a

copy.

While many disk nibbler type copiers will have no problem making

a full copy of this disk, it cannot be copied by simply using the

DOS DISKCOPY program.

An example of a broken zone system may again start with a

standard DOS disk type, D4.  this normally stops at track 39.

You can then define zone 2 as being something on track 41.  Or

you could select a single sided disk type (say D1) then put a

track in the middle of side 1.  (Of course, this disk must be

used in a double sided drive to be able to read the track on the

back side.)

15

FreeForm v2.3                                        Users' Guide

-----------------------------------------------------------------

Saving the Parameters

After editing and/or creating new parameter sets, you will

probably want to save them for future use.  This is done by

pressing the <F1> key when in EDIT mode.  FreeForm saves its

parameter values in a file named 'FREE23.DEF' on the current

system default drive (i.e. the drive from which it was loaded

itself).

SETTING DEFAULTS

FreeForm lets you set various program defaults to suit your

needs.  To reach the setup screen, press the <F3> key from the

main menu.

You can change the screen colors, the system path (used for

copying DOS boot files) and the DOS boot messages.  On screen

prompts will help guide you during the process.

Two boot messages are used: the first message is displayed when a

DOS system disk is being booted.  It will typically say something

like "Booting System".  The second message is an error message

that is displayed when an attempt is made to boot from a non-

system disk, typically "Non-system disk or boot error.  Replace

and re-boot".  For both messages, you can insert a carriage

return with ~ and a line feed with ^.  This allows you to put a

message on more than one line.  The first message can be a

maximum of 44 characters long and the second message up to 60.

Press <F1> to save the new defaults, or <Esc> to return to the

main menu without saving.

EXITING FREEFORM

To exit FreeForm and return to DOS, press the <F10> key from the

main menu screen.  If FreeForm detects that you have edited the

parameter file since last saving it, you will be given the chance

to save the new values before exiting.

16

FreeForm v2.3                                        Users' Guide

-----------------------------------------------------------------

USING SMAX.COM

While DOS versions 3.2x and later can automatically handle the

housekeeping required to access MAXI style extended capacity

disks, the BIOS and hardware on many systems will not. SMAX.COM

is a memory resident driver which is used to improve the

compatibility of MAXI extended capacity disks (disk types D9 to

D14) with some combinations of hardware and DOS. It is not a true

device driver in the conventional sense of DOS terminology.

Rather, it is a simple TSR (terminate but stay resident) program

which patches into the BIOS disk interrupt vector (interrupt hex

13) and changes the EOT (end of track, or last sector in a track)

parameter to match the increased value of the MAXI extended

capacity disks when it detects a disk read or write operation.

MAXI Form v1.8, MAXI Disk v3.2, and FreeForm v2.3 will

automatically configure SMAX for the type of disk being

formatted.  SMAX will automatically configure itself for the

read/write operations to allow transparent access to all DOS and

MAXI formatted disks, regardless of density.

* * * *

SMAX MAY BE FREELY COPIED AND DISTRIBUTED TO YOUR

FRIENDS FOR THEIR PRIVATE, NON-COMMERCIAL USE (SO THAT

THEY CAN READ FILES WHICH YOU MAY SEND THEM ON MAXI

FORMATTED DISKS) PROVIDED THAT THE COPYRIGHT NOTICE IS

NOT REMOVED. THIRD PARTY SOFTWARE DEVELOPERS OR VENDORS

WISHING TO INCLUDE SMAX WITH THEIR OWN COMMERCIAL

PRODUCTS REQUIRE THE PRIOR WRITTEN PERMISSION OF HERNE

DATA SYSTEMS ALONG WITH A RUNTIME DISTRIBUTION LICENSE.

* * * *

When to Use SMAX

To determine whether or not you need to use SMAX with your system

configuration, perform the following simple test.

-  Install SMAX on a cold system boot.

-  Format an extended capacity (420k/800k/1.4 meg/1.6 meg/2.8

meg) disk with FreeForm, using the largest capacity floppy

drive on your system.

17

FreeForm v2.3                                        Users' Guide

-----------------------------------------------------------------

-  Turn off SMAX or reboot your system without SMAX installed,

then try to copy a number of relatively long files (eg. over

about 20k bytes long each) to the new disk using the DOS

COPY or XCOPY command or equivalent.

-  If you get an error message (eg. 'general failure reading

drive ...' or 'sector not found') then you need to install

SMAX to use the MAXI formatted extended capacity disks.

* * * *

IF SMAX IS REQUIRED ON YOUR SYSTEM, IT SHOULD BE

INSTALLED BEFORE USING MAXI FOR FORMATTING THE EXTENDED

CAPACITY DISKS AND BEFORE READING OR WRITING ANY

PREVIOUSLY FORMATTED MAXI DISKS.  THIS IS MOST EASILY

DONE BY INSERTING THE REQUIRED COMMAND LINE INTO YOUR

AUTOEXEC.BAT FILE.

* * * *

Installing SMAX

* * * *

BECAUSE IT IS A TERMINATE-AND-STAY-RESIDENT PROGRAM, DO

NOT INSTALL SMAX FROM A DOS WINDOW (UNDER MS WINDOWS)

OR THROUGH A MENU SHELL SUCH AS DOS 5 OR 6 DOSSHELL.

IF YOU DO, YOUR SYSTEM MAY LOCK UP WHEN YOU TRY TO

ACCESS ANY KIND OF DISK (INCLUDING THE HARD DRIVE).

HOWEVER, ONCE SMAX HAS BEEN INSTALLED, YOU CAN CHANGE

ITS CONFIGURATION FROM A DOS WINDOW OR THROUGH A MENU

SHELL.

* * * *

To install SMAX automatically on a cold boot, type the following

command at the DOS prompt (or insert the line into your

AUTOEXEC.BAT file):

SMAX <--

SMAX checks to see that you are using DOS 3.20 or later before

installing itself.  Once installed, SMAX remains resident in your

computer's RAM and is active for all floppy drives on your

system. The resident portion of SMAX requires only 1.5k bytes of

memory.  SMAX also accepts several command line options. These

18

FreeForm v2.3                                        Users' Guide

-----------------------------------------------------------------

are:

SMAX 0    Turns off SMAX so that it remains hooked into the

interrupt 13h chain, but does nothing.  If SMAX is not

resident, an error message is displayed. Note that the

'0' can be either the letter o (uppercase or lowercase)

or the numeral zero.

SMAX 1    Turns on SMAX that has been previously turned off with

the 0 option.  If SMAX is not resident, it is installed

as for a first time installation. Note that the '1' can

be either the letter i (uppercase or lowercase) or the

numeral one.

SMAX 40   Turns off the 80 track double density mode for 1.2 meg

drives to allow them to access 360k/420k disks.

SMAX 80   Turns on the 80 track double density mode for 1.2 meg

drives.  This allows 720k/800k 5-1/4 inch disks to be

formatted and used in 1.2 meg drives with normal 360k

DD media.  On most systems, this feature is only

supported for drives A: and B: due to BIOS limitations.

In addition, it will also remove the ability for these

drives to access 360k/420k disks.

SMAX A    Sets SMAX to automatic 40 track/80 track sensing mode

(where it can read/write either 40 or 80 track double

density disks interchangeably).  This is the default

mode.

SMAX E    Enables SMAX for secondary controlled drives.  This is

the default mode.

SMAX N    Disables SMAX for secondary controlled drives.  The

BIOS in some secondary floppy disk controller cards,

such as those used with some external expansion drives,

is not compatible with SMAX.  This options allows SMAX

to be used for internal, primary controlled drives,

while not interfering with the use of secondary

controlled drives.

SMAX ?    Checks the current status of SMAX in memory, if

present.

19

FreeForm v2.3                                        Users' Guide

-----------------------------------------------------------------

* * * *

SMAX 40 AND SMAX 80 ARE NOT REQUIRED WHEN USING MAXI

FORM v1.8, MAXI DISK v3.2, OR FREEFORM v2.3 OR LATER.

THEY ARE PROVIDED FOR BACKWARD COMPATIBILITY WITH

EARLIER VERSIONS ONLY.

* * * *

If you attempt to install SMAX with no options specified (or with

the ? option) when it is already resident, a status message will

be displayed showing the currently active options. If you attempt

to install SMAX with a command line option not listed above, you

will get an error message.

SMAX options can be changed at any time after it is resident in

memory by running the appropriate command line. This will change

the current parameters for the copy of SMAX in memory and will

not result in another copy of SMAX being loaded into memory.

SMAX can be loaded into high memory with the DOS 5 or 6 LOADHIGH

(LH) command, provided that your CONFIG.SYS includes the proper

XMS and EMM memory managers.  (See your DOS manual for more

information on the LOADHIGH command).

Disabling SMAX

SMAX will not normally interfere with the use of standard floppy

DOS disks and it is completely transparent to hard disks. Both

regular style DOS disks and MAXI extended capacity disks can be

used interchangeably and transparently on any floppy drive in

your system by most application software such as word processors,

spreadsheets and databases. However, it is recognized that some

programs, such as those which access the disk at the physical

sector level or in an unconventional manner (including some hard

disk fast backup utilities and some versions of the DOS FORMAT

and DISKCOPY programs) do not work correctly with SMAX installed.

For such cases, SMAX can be de-activated without re-booting the

computer by using one of two command line options as summarized

above:

SMAX 0 <--

The 0 option will set a disabling flag within SMAX itself,

without actually removing the code from the interrupt 13h chain.

SMAX can then be re-activated using the 1 option when required.

A special compatibility concern may arise when using two floppy

disk controller cards: a primary card and a secondary or add-on

20

FreeForm v2.3                                        Users' Guide

-----------------------------------------------------------------

card.  The primary card may require SMAX to work correctly, while

the BIOS in the secondary card (such as a CompatiCard, usually

controlling external floppy drives) may become confused by SMAX.

In this case, use the SMAX N option to disable SMAX for

externally controlled drives.

The SMAX R and SMAX U options to unhook and re-hook SMAX into the

interrupt chain are no longer supported in SMAX v4, since they

are no longer required for reliable operation.

80 Cylinder Double Density Mode

The 1.2 meg disk drives used in AT class machines are capable of

reading and writing 80 cylinders. Normally this feature is used

only in high density mode (1.2 meg or 1.4 meg).  However, in

order to do this reliably, special (i.e. more expensive) high

density (HD) diskettes are required.  Some BIOSes will let you

format and use 80 cylinder double density (DD) 5-1/4 inch disks

in 1.2 meg drives.  However, most will not.  With the 80 option

of SMAX, you can take advantage of this to get up to 800k on a

normal 360k double density disk with any BIOS! This is done by

recording the data at the same bit density as normal 360k disks,

but with twice the number of cylinders.  In terms of reliability,

the disks are just as reliable as 360k disks that have been

formatted and used in 1.2 meg drives.

Of course, the 80 cylinder DD disks will NOT be usable in 360k

drives, but will be usable in the 80 cylinder DD, 5-1/4 inch

drives used on some non-standard MS-DOS systems.  The 80 option

is not required if your BIOS normally supports reading and

writing this disk type.

To activate the 80 cylinder mode of SMAX, use the command line:

SMAX 80 <--

(Because of limitations of BIOS flag storage areas, this feature

is limited to drives A: and B: only.  On most systems this is

adequate because the standard AT type machine has only two drives

with the 1.2 meg drive as A:.)  The 80 cylinder mode will not

affect normal 80 cylinder disks (i.e. 3-1/2 inch and 1.2/1.4 meg

5-1/4 inch).  However, with the 80 cylinder mode activated, you

will not normally be able to access 360k/420k disks in the 1.2

meg drive.  To de-activate the 80 cylinder mode, use the command

line:

SMAX 40 <--

21

FreeForm v2.3                                        Users' Guide

-----------------------------------------------------------------

This will return the machine to normal 40 cylinder DD mode for

1.2 meg drives.  To put SMAX into auto 40/80 mode (the normal

default), use:

SMAX A <--

Note that MAXI Form v1.8, MAXI Disk v3.2, and FreeForm v2.3 (the

three disk formatting utilities designed to work with SMAX) will

automatically configure SMAX to the correct settings without the

use of the 40/80/A command line switches.  When in the default

automatic mode, SMAX will configure itself for normal disk reads

and writes.

* * * *

SMAX HOOKS ITSELF INTO BIOS INTERRUPT HEX 13 (THE DISK

CONTROLLER INTERRUPT) AND USES INTERRUPT HEX 7E (NOT

NORMALLY USED BY THE BIOS, DOS OR OTHER KNOWN PROGRAMS)

FOR ITS OWN INTERNAL FUNCTIONS.

* * * *

THE TECHNICAL DETAILS

FreeForm works by using the various functions of the BIOS disk

service interrupt hex 13.  Specifically, function 5 is used for a

track by track format, function 1 is used to read sectors and

function 2 is used to write them.  Functions hex 17 and 18 are

used to set the characteristics of the high density drives

(1.2/1.44/2.8 meg) prior to formatting.  These functions are not

implemented on PC and XT class machines with low density drives

(360/720 k) only.

Technical Data of MAXI Style Disks

DISK TYPE

------------------------------------------------

PARAMETER           420k  800k  1.4meg    1.6meg   1.7meg  3.2meg

-----------------------------------------------------------------

sectors/track        10    10     17       20       21      39

default # tracks     42    80     82       82       82      82

# FAT sectors         2     3      9       10       10      10

sector interleave     1     1      1        1        1       1

-----------------------------------------------------------------

22

FreeForm v2.3                                        Users' Guide

-----------------------------------------------------------------

APPENDIX A: ENHANCEMENTS AND IMPROVEMENTS

New Features of v2.3

FreeForm v2.3 has the following new features and improvements

over v2.2:

-  FreeForm is now supported as a shareware product.

-  Automatic detection of drive type eliminates need for manual

installation of program.

-  The formatting process can be user terminated by pressing

the <Esc> key at any time during the formatting process.

-  Ability to modify standard DOS disk parameters such as

number of directory entries, boot messages and allocation

unit size.

-  Program setup/configuration routine now built into main

program.

-  Automatic interface with SMAX to set required parameters for

different types of disks without SMAX command line switches.

-  Improved SMAX driver (v4) features automatic 40/80 track

support; ability to be loaded into high memory with DOS 5/6

LOADHIGH; improved compatibility with DOS FORMAT and

DISKCOPY programs; and streamlined interface with fewer

command line parameters (U and R switches no longer

supported, 40 and 80 switches maintained, but not required

for FreeForm v2.3).

23