BzztBzzt!

Installation

&

Hardware

Copyright © 1994  Walt Stoneburner
All Rights Reserved
Distributed by Wizard Workshop and Company

How to Identify and Use
Your BzztBzzt! Box

Table of Contents

List of Pieces

System Requirements

Hardware Installation

# List of Pieces

The following parts should be included in your BzztBzzt! package:

- ( 1 )		BzztBzzt! game box

- ( 4 )		remote game buttons with 25 feet of cord each

- ( 1 )		computer cabel extension, 6 feet in length

- ( 1 )		BzztBzzt! software disk

# System Requirements

To operate the BzztBzzt! system, you must have the following:

- Game (joystick) port

- 386 system or better		(recommened)

- VGA monitor or better	(recommended)

# Hardware Installation

Connect the grey (6 foot) cabel to the computer and the other end of the cabel to the blue BzztBzzt! box.  The male end of the cabel has 15 pins and goes into your game port.  The female connector connects to the BzztBzzt! box.

Note: On some machines, part of  the game port might be obstructed by the machine’s chasis.  If the cord will physically not plug in due to an obstruction, you may remove the plastic shielding and slide it up the cord, away from the connector with a small flathead screwdriver.  One end of the metal connector has been removed to accomodate machines with weird cases.

Female Connector

Male Connector

Back of BzztBzzt! box.  (Male)

# The BzztBzzt! Box and Remote Units

The top of the box has screws and the bottom of the box has four little plastic stub feet.

In the back is a 15 pin connector for connecting to the computer.

Top of box

On the front are four 1/8th of an inch jacks.  Remotes are firmly plugged into the box in these jacks.  Never force a jack in, and do not insert any thing other than a remote buttons box into the jacks.  Doing so could result in damage to the device being plugged in, the components in the box, or worse, your computer.

Front of box

Jack (female)

Player Button

Jack (male)

Game Remote

The end with pins

The end with holes